Pfu DNA Polymerase

Tech : maozhang2000@hotmail.com

Tel: 8610-82796972  Mob: 13691030050

Web: www.aidlab.cn

Aidlab Biotech

Technical Manual

| Components | PC0302 | PC0303 |
| --- | --- | --- |
| Pfu DNA Polymerase(5 U/μl) | 100 μl | 600 μl |
| 10X Pfu Buffer (with MgSO4) | 1 ml | 6 ml |
| SuperPure dNTP mixture (10mM each) | 200 μl | 1.2 ml |

| Concentration: 5 U/μl Cat # | Size |
| --- | --- |
| PC0302 | 500U |
| PC0303 | 3000U |

Store: －20C , 1 year

Description

Pfu DNA Polymerase is isolated from E.coli strain that contains Pyrococcud fureosis DNA Polymerase gene.  Pfu DNA Polymerase possesses a proofreading 3'-5' exonuclease activity. Pfu DNA Polymerase is recommended for use in PCR and primer extension reactions that require high-fidelity synthesis. The accuracy of Pfu DNA Polymerase is approximately 8-fold higher than Taq DNA Polymerase. Pfu DNA Polymerase-generated PCR fragments are blunt-ended. The extension rate is about 0.5 kb/min.

Unit Definition

One unit of EasyPfu DNA Polymerase incorporates 10 nmol of deoxyribonucleotide into acid-precipitable material at 74C for 30 min.

Storage Buffer

50 mM Tris-HCl (pH 8.0), 50 mM KCl, 1 mM DTT, 0.1 mM EDTA, stabilizers, and 50% (v/v) glycerol.

10X Pfu Buffer with 20 mM MgSO4

200 mM Tris-HCl (pH 8.8), 100 mM (NH4)2SO4, 100 mM KCl, 1% (v/v) Triton X-100, 1mg/ml BSA and 20 mM MgSO4.

Notes

- Because of its 3'-5' exonuclease activity, the extension rate of Pfu DNA polymerase is lower than that of Taq DNA polymerase. Therefore, during the extension step, please set extension time according to the product length.

- High primer purity and concentration (about 0.2-0.5μM) required . The 3'-5' exonuclease activity may degrade primers, especially ,when the reaction mixture lack of dNTP. To overcome the degradation, add the Pfu last into the reaction mixture and carry PCR reaction at once.

- The stability of Pfu DNA Polymerase is higher than Taq DNA Polymerase. For the CG rich template, the denaturation temperature can increase to 98°C without affecting the activity of Pfu DNA polymerase.

- Pfu DNA polymerase produces blunt-ended amplification products. The products can be cloned by blunt-ended vector not by T/A vector.

PCR mixture set up (for 50 μl reaction volume)

| Components | Volume | Final Concentration |
| --- | --- | --- |
| Template DNA | Variable | as required |
| Forward Primer (10 μM) | 1-2 μl | 0.2-0.4 μM each |
| Reverse Primer (10 μM) | 1-2 μl | 0.2-0.4 μM each |
| 10X Pfu Buffer (contains Mg2+) | 5 μl | 1x |
| 10 mM dNTPs | 1 μl | 0.2 mM |
| Pfu DNA Polymerase | 0.5 μl | 2.5 units |
| ddH2O to final volume | 50 μl | Not applicable |

PCR Condition

94°C   2-5 min

94°C   30 sec

50-60°C 30 sec 30-35 cycles

72°C 0.5 kb/min

72°C 5-10 min