◆ TRIpure Reagent 抽提指南

◆ 目录号 RP02

◆ 使用手册

◆ 实验室使用，仅用于体外

◆ TRIpure Reagent 抽提指南

◆ 目录号 RP02

◆ 使用手册

◆ 实验室使用，仅用于体外

◆ TRIpure Reagent 抽提指南

◆ 目录号 RP02

◆ 使用手册

◆ 实验室使用，仅用于体外

◆ BCA Protein Assay Kit

◆ Cat# PP01

◆ Manual

◆ For laboratory research use only

BCA Protein Assay Kit

Cat#：PP01

| Cat# | Preps |
| --- | --- |
| PP0101 | 200 |
| PP0102 | 500 |
| PP0103 | 2500 |

- Content Storage and Stability:

| Content | Storage |  | PP0101 | PP0102 | PP0103 |
| --- | --- | --- | --- | --- | --- |
| BSA standards （5mg/ml BSA） |  | -20℃ | 0.5ml | 0.5ml | 1ml×3 |
| Solution A |  | RT | 40ml | 50ml×2 | 250ml×2 |
| Solution B |  | RT | 1ml | 2ml | 5ml×2 |

All reagents are stable at least one year when stored properly

- Principle

The BCA Protein Assay combines the well-known reduction of Cu2+ to Cu1+ by protein in an alkaline medium with the highly sensitive and selective colorimetric detection of the cuprous cation (Cu1+) by bicinchoninic acid. Two molecules of BCA chelate with one cuprous ion forming purple-coloured product. The BCA/copper complex is water-soluble and exhibits a strong absorbance at 562nm. Protein concentration can be determined based on absorbance because of the linear relation between them.

- Features

- Faster and easier than the Lowry method.

- Sensitive, The minimum detection level of 25μg/ml.

- Compatible with most detergents(SDS/TritonX-100/ Tween20/Tween60/Tween80 up to 5%).

- Good linear range from 20-2000μg/ml.

- Coefficient of variation lower than Bradford.

### Notes

- Please mix protein standards after total dissolved first, and then diluted into a series of different concentrations of protein standards.

- When Reagent B is first added to Reagent A, turbidity is observed that quickly disappears upon mixing to yield a clear, green solution.

- A microplate Spectrophotometer(wavelengh540-590nm) or

Spectrophotometer is needed. When carry on spectrophotometer, adjust the volume of the solution A, B and sample based on the quartz cell.

- BCA assay is interfered by chelating agents and high concentration reducing agents. Make sure EDTA<10mM, no EGTA, DTT<1mM and β-ME <1mMin sample buffer. When BCA assay can not work, resolve it by dialyze, desalt, dilute sample or precipitate proteins with acetone or trichloroacetic acid (TCA).

- Heat by microwave oven can quicken the process, but make sure the temperature is not too high.

- If the background noise of dilution or lysis solution is high, please use the Aidlab Bradford Assay kit.

- Procedure

- Prepare working solution: Mix solution A with solution B on volume ratio 50:1.

Prepare sufficient working solution based on the number of samples to be assayed. The Working solution is stable for 24h at room temperature.

- Dilute BSA standard to 0.5mg/ml with 0.9%NaCl or PBS.

- Pipette 0, 1, 2, 4, 8, 12, 16, 20µl BSA standard into microplate wells and make up all to 20μl with the diluted solution for standard protein.

- Pipette 20µl sample into microplate wells.

If the volume of sample less than 20µl, please make up to 20μl with the diluted solution for standard protein.

- Add 200µl BCA working solution to each well and mix plate thoroughly. Incubate at 37℃ for 30-60min or at room temperature for 2h or at 60℃ for 30min.

Increasing the incubation time or temperature increase the absorbance and decrease the minimum detection level.

- Cool plate to room temperature and measure the absorbance at 562nm or 540nm between 590nm on a plate reader.

- Use the standard curve to determine the protein concentration.