TRUEscript Green

Tech : maozhang2000@hotmail.com

Tel: 8610-82796972  Mob: 13691030050

Web: www.aidlab.cn

Aidlab Biotech

One Step qRT PCR Kit

Technical Manual

| Components | PC4101 | PC4102 |
| --- | --- | --- |
| 2 XqRT-PCR Mix (Contains Sybr Green I) | 1.25 ml | 2x1.25 ml |
| TRUEscript OneStep Enzyme Mix | 100 μl | 200 μl |
| RNase free H2O | 1.5 ml | 1.5 ml |

| Cat # | Size |
| --- | --- |
| PC4101 | 50 applications |
| PC4102 | 100 applications |

Store：－20C , 1 year

Description

TRUEscript Green One-Step qRT-PCR Kit combines the first strand cDNA synthesis and qPCR in the same tube to simplify reaction setup and reduces the possibility of contamination. Only gene specific primers can be used for this kit. TRUEscript Green One-Step qRT-PCR Mix contains all the necessary reagents for cDNA synthesis and qPCR except total RNA/mRNA template and gene specific primers.

• High efficiency cDNA synthesis with TRUEscript OneStep Enzyme Mix and 2 XqRT-PCR Mix (Contains Sybr Green I), followed by PCR amplification with resulting cDNA. Simple procedure helps to minimize contamination.

• cDNA synthesis and qPCR are performed in a single tube using gene specific primers with total RNA or mRNA as templates.

One step qRT-PCR mixture set up (for 25, 50 μl reaction volume)

Note: Melting 2×qRT-PCR Mix tube and mix well before use. You can put in 4C in short time if use frequently

1. Add Componets according to table on ice:

| Components | Volumn(25μl) | Volumn(50μl) | Final Concentration |
| --- | --- | --- | --- |
| 2×qRT-PCR Mix | 12.5 μl | 25 μl | 1× |
| Forward Primer（10μm） | 0.5 μl | 1 μl | 0.2 μM |
| Reverse Primer（10μm） | 0.5 μl | 1 μl | 0.2μM |
| Enzyme Mix | 1 μl | 2 μl | - |
| RNA template | X μl | X μl | 1ng-1μg |
| RNase free H2O to final volume | 25 μl | 50 μl | Not applicable |

One step qRT-PCR ：
1. Mix well gently and briefly centrifuge.

2. Adjust PCR machine to 42C, then put tube into PCR to run following program.
Program:

|  | Temperature | time | cycle |
| --- | --- | --- | --- |
| 1 | 42C | 20-30 Min | 1 |
| 2 | 94C | 2-3 Min | 1 |
| 3 | 94C | 20 Sec | 35-40 |
|  | 55-60C | 20 Sec |  |
|  | 72C | 20 Sec |  |
| 4 | Melting curve analysis |  |  |

Note: this program is three step PCR method. you also can use two step PCR method.

Note

- High quality RNA template is recommended for use to ensure successful cDNA synthesis.

- This kit is only suitable for Gene Specific Primer(GSP), but unsuitable for first-strand cDNA synthesis using Oligo(dT) or Random Primer.

- This kit doesn't contain Rox Reference Dye, if you need in some real-time PCR machine. You can order Rox Reference Dye from us (cat# PC38).